PMSM Parameter Identification Experiment Plan

# Base Parameters

| - Motor speed constant | - KV | - 210rpmV |
| --- | --- | --- |
| - Pole pairs | - p | - 14 |
| - Line-to-line resistance | - RLL | - 42.5 mΩ |
| - Stator radius | - r1 | - 31mm |
| - Motor radius | - r2 | - 34,4mm |
| - Motor mass | - mmotor | - 331g |

# A: Identification Phase Resistance

Rs=RLL2=42.5 2 mΩ=21.25 mΩ

# B: Identification Flux Linkage

- Calculate torque constant.

Kt=602∙π∙KV=602⋅π⋅210rpmV= 0,04547NmA

- Calculate flux linkage.

ψPM=Kt1.5⋅p=0,04547NmA1.5⋅14=2.165 mWb

# C: Identification Stator Inductance

- Measure line to line inductance.

LLL,ab= 40,27uH

LLL,ac=31,78uH

LLL,ac=32,80uH

LLL=34,95uH

- Calculate stator inductance.

Ls=LLL2=17,475uH

# D: Identification Rotor Inertia

- Obtain rotor geometry.

r1=31mm,  r2=34,4mm,

m=120g (≈36%⋅mmotor, assumption)

- Calculate rotor inertia.

J=12mr12+r22=1,29×10-4kg⋅m2

# E: Identification Viscous Friction Coefficient

- Leave all phases open and connect oscilloscope.

- Externally spin motor.

- Measure back-EMF line-to-line voltage during deceleration.

- Convert back-EMF to rotor speed.

ωt=2π60⋅Kv⋅VLL(t)

- Find maxima and minima.

- Decide which are better suited for fit.

- Take log and fit straight line.

- Determine slope.

BJ= -slope= 6.677861s

- Calculate viscous friction coefficient.

B=J⋅BJ= 0.000861444Nmsrad

# Final Parameter Set

| Parameter | Value |
| --- | --- |
| Rs | 21.25 mOhm |
| Ls | 17.475 uH |
| psi_pm | 2.165 mWb |
| J | 1.29*10^-4 kg*m^2 |
| B | 0.000861444 Nms/rad |